CERTIFICATE OF ANALYSIS

| Product Name: Vonoprazan Fumarate |  |  |
| --- | --- | --- |
| Batch No: YSC-1519-2601001 |  |  |
| Manufacture Date: 2025-AUG-05 | Exp Date: 04/01/2030 |  |
| Test Items | Specifications | Test Result |
| Appearance | White or off-white crystalline powder | Conforms |
| Solubility | Soluble in dimethyl sulfoxide, sparingly soluble in N,N-dimethylacetamide, slightly soluble in methanol and in water, practically insoluble in acetonitrile and in ethyl acetate | Conforms |
| Identification | A. HPLC:The retention time of the major peak of the Sample solution corresponds to that of the Reference solution, as obtained in the Assay. B. IR:Compare the infrared absorption spectrum of vonoprazan fumarate and that of the Reference Standard: both spectra exhibit similar intensities of absorption at the same wave numbers. | Conforms |
| Water | ≤0.5% | 0.01% |
| Residue on ignition | ≤0.1% | 0.03% |
| Heavy metals | ≤10ppm | Conforms |
| Related substances | Impurity D:≤0.15% | 0.04% |
|  | Impurity E:≤0.15% | Not detected |
|  | Impurity F:≤0.15% | 0.03% |
|  | Impurity I:≤0.15% | Not detected |
|  | Impurity O:≤0.30% | 0.04% |
|  | Other single impurity:≤0.1% | 0.02% |
|  | Total impurities:<1.0% | 0.13% |
| Residual solvents I | Acetonitrile:≤0.041% | Not detected |
|  | Methanol:≤0.3% | 0.01% |
|  | Dichloromethane:≤0.06% | Not detected |
|  | Tetrahydrofuran:≤0.072% | Not detected |
|  | Ethyl acetate:≤0.5% | Not detected |
| Residual solvents 2 | Triethylamine:≤0.5% | Not detected |
|  | N,N-Dimethylacetamide:≤0.109% | Not detected |
| Assay | ≥98.5% (Anhydrous basis) | 99.7% |
| PSD | D90≤50μm | 49.9μm |
| CONCLUSION | The Product Complies with The Requirements of The Above Specifications. |  |