CERTIFICATE OF ANALYSIS

| Name Product: Cefuroxime Axetil Coated |  |  |  |  | Batch No: CACK260003 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Analytical Report No. UR-2600291 |  |  |  |  | Mfg. date: MAY 2026 |  |  |
| Quantity 185.00 KG |  |  |  |  | Exp. date: APR 2029 |  |  |
| NO | TESTS |  |  | SPECIFICATIONS |  |  | RESULTS |
|  | Appearance |  |  | Almost White to Off-White powder |  |  | Almost white powder |
|  | Solubility |  |  | Freely soluble in acetone insoluble in ether and in water. |  |  | Freely soluble in acetone insoluble in ether and in water. |
|  | Identification |  | By IR | The IR Spectrum obtained of the sample should exhibit same spectrum as that obtained of a similar preparation of cefuroxime Axetil reference standard/Working Standard |  |  | The IR Spectrum obtained of the sample should exhibit same spectrum as that obtained of a similar preparation of cefuroxime Axetil reference standard/Working Standard |
|  |  |  | By HPLC | The principal peaks in the chromatogram obtained with the test solution corresponds to the peaks due to Diastereoisomer A and B in the chromatogram obtained with reference solution in test parameter assay. |  |  | The principal peaks in the chromatogram obtained with the test solution corresponds to the peaks due to Diastereoisomer A and B in the chromatogram obtained with reference solution in test parameter assay. |
|  | Diastereoisomer ratio |  |  | Between 0.48 and 0.55 |  |  | 0.52 |
|  | Related Substances By HPLC | Cefuroxime Axetil D3 isomers |  | Not more than 1.5% |  |  | 0.52% |
|  |  | Cefuroxime Axetil E isomers |  | Not more than 1.0 % |  |  | 0.07% |
|  |  | Any Other secondary peak |  | Not more than 0.5% |  |  | 0.25% |
|  |  | Total impurities (Excluding Known impurities) |  | Not more than 3.0% |  |  | 0.41% |
|  | Acetone by GC |  |  | Not more than 1.1% |  |  | 0.12% |
|  | Water by KF |  |  | Not more than 1.5% |  |  | 0.56% |
|  | Assay by HPLC (On anhydrous basis) |  |  | Not less than 60% w/w and Not more than 70% w/w on anhydrous basis. |  |  | 68.8% |
| Additional Test: |  |  |  |  |  |  |  |
| Bulk density |  |  |  | For information only |  | 0.37 g/ml |  |
| Tapped density |  |  |  | For information only |  | 0.48 g/ml |  |
| Sieve analysis |  |  |  | Should be pass 100% through #18 mesh |  | 100 % |  |
| Remarks: The product conforms to in house specification i.e STS-FCACH-01 with respect above test. |  |  |  |  |  |  |  |

THIS CERTIFICATE WAS ISSUED ELECTRONICALLY AND DOES NOT REQUIRE A STAMP.