CERTIFICATE OF ANALYSIS

| Packing: -- | Date of Analysis: 28/02/2026 |
| --- | --- |
| Invoice No: -- | Batch No: KALM26001A |
| CAS No: 432-60-0 | Mfg. Date: 25-02-2026 |
| Retest Date : 24-02-2030 | Quantity: 18.00 Kg |

ITEM: ALLYLESTRENOL MICRONIZED IH

| TEST | Specification | Result |
| --- | --- | --- |
| Description | White to almost white crystalline powder, odorless | Almost white crystalline powder, odorless |
| Solubility | Practically insoluble in water, very soluble in ethanol, chloroform, acetone and in dioxane. Freely soluble in methanol and in ether | Complies |
| Identification | - By IR: IR spectrum should match with the that of standard | Complies |
|  | - By HPLC: HPLC RT corresponds to that of standard | Complies |
|  | - By TLC TLC-RF value identical with that of standard | Complies |
| Melting point | Between 78.0° and 81.0°C | 79.0°C |
| Specific optical rotation: |  |  |
| - 1% solution in 1,4- Dioxane | Between +43.0° and + 47.0° | +45.3° |
| - 1% solution in Chloroform | Between +37.0° and + 40.0° | +37.4° |
| Loss on drying (Over P2O5 for 5 hours) | Not more than 0.5% w/w | 0.26% w/w |
| Residue on Ignition | Not more than 0.1% w/w | 0.03% w/w |
| Purity Test: |  |  |
| - Clarity & Color (2% solution in Dioxane) | The solution is clear and colorless | Complies |
| - Other steroids by TLC | A single brown spot appears in sample chromatographic plate | Complies |
| Assay by HPLC (On dried basis) | NLT 97.0% w/w and not more than 102.0% w/w | 99.9% w/w |
| Residual solvents by GC-HS#: |  |  |
| Diethyl ether | Not more than 5000 ppm | <1.99ppm (DL) |
| Acetonitrile | Not more than 410 ppm | 90 ppm |
| Heavy Metals | Not more than 20 ppm | Less than 20ppm |
| Particle size (By laser diffraction) D90 | Informative | 6.8 µm |

Storage Condition: Do not store above 30°C, protect from light.

Analyses By                                                                                              Approved By

(QC Executive)                                                                                          (QC Manager)