Elder House, Plot No C-9, Dalia Industrial Estate, Off Veera Desai Road, Mumbai, Maharashtra,400053

Certificate of Analysis

| Name of the Product Fluticasone Propionate BP (Micronized) |  |  | Batch No APL/G/FLP/26003 |  |
| --- | --- | --- | --- | --- |
| Analytical Report No. EL/QC/26/07/07 |  |  | Mfg. date May 2026 |  |
| Quantity dispatched/Sample Quantity 5.00 kg |  |  | Exp date Apr 2031 |  |
| Result of Analysis |  |  |  |  |
| No | Tests parameters | Specifications |  | Results |
|  | Description | White or almost white micronized Powder |  | Complies |
|  | Identification | Sample IR should concordance with standard IR |  | Complies |
|  | Solubility | Practically insoluble in water, sparingly soluble in Methylene chloride, slightly soluble in ethanol. (96%). |  | Complies |
|  | Specific Optical Rotation | +32° TO +36° |  | +33.84° |
|  | Acetone Limit | Not More Than 1.0 % |  | 0.002 % |
|  | Water Content | Not More Than 0.5 % |  | 0.22 % |
|  | Related Substance by HPLC | Impurity C Not more than 0.2 % |  | 0.02 % |
|  |  | Impurity D Not more than 0.3 % |  | Not Detected |
|  |  | Impurity G Not more than 0.3 % |  | Not Detected |
|  |  | Impurity J&K Not more than 0.15% |  | Not Detected |
|  |  | Any unspecified Impurities Not more than 0.1 % |  | 0.04 % |
|  |  | Total Impurities Not More Than 0.8 % |  | 0.11 % |
|  | Assay by HPLC | 97.5 % TO 102.0 % |  | 99.77 % |
|  | Particle size | 95% less than 5.0 Microns |  | 3.76 μm |
|  |  | 100% less than 10.0 Microns |  | 7.35 μm |
|  | Residual Solvent | Acetone: Not more than 5000 ppm |  | 26 ppm |
|  |  | Tetrahydrofuran: Not more than 720 ppm |  | Not Detected |
|  |  | Toluene: Not more than 890 ppm |  | 6 ppm |
| Remarks: Complies with the BP and In-house. |  |  |  |  |

Page 1 of I