| CERTIFICATE OF ANALYSIS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Product Name: Nebivolol Hydrochloride INH Batch No . : CQ/NEB/2022/012 |  |  | Date of Mfg: Septemper-2022 Date of Exp: August -2025 |  |
| Sr.No. | Testing parameters | Specification |  | Result |
| 1. | Description | White or almost white powder |  | White to off white powder. |
| 2. | Solubility | Sparingly soluble in Methanol. |  | Sparingly soluble in Methanol. |
| 3. | Identification by |  |  |  |
|  | By IR | The infrared absorption spectrum of the substance being examined should be concordant with the spectrum obtained from Nebivolol Hydrochloride Working Standard. |  | Complies |
|  | By HPLC | Retention time of the D and L isomers in the sample should match with that of working standard during Chiral purit by HPLC. |  | Complies |
|  | By Chemical | Complies for the test for chloride |  | Complies |
| 4. | Specific optical rotation | Between – 1.0˚ and +1.0˚. |  | 0.3 |
| 5. | Related substances by HPLC Any other individual Impurity Total Impurities | NMT 0.5% NMT 1.0% |  | 0.06% 0.09% |
| 6. | Heavy Metals | NMT 10 ppm |  | Less than 10 ppm |
| 7. | Chloride Content (% w/w on anhydrous basis) | Between 7.5% w/w and 8.5% w/w. |  | 8.0% w/w |
| 8. | Chiral Purity (By HPLC) - D-isomer - L-isomer - Any other individual impurity - Total of D and L isomers | - Between 48.5% and 51.5% - Between 48.5% and 51.5% - Not more than 0.15% - Not less than 99.00% |  | - 50.48% - 49.26% - 0.07% - 99.8% |
| 9. | Water content | Not more than 0.50% w/w. |  | 0.09% w/w |
| 10. | Sulphated Ash (%w/w) | Not more than 0.10% w/w. |  | 0.06% |
| 11. | Assay (By Potentiometry ) | 98.5% to 101.5% w/w on dried basis |  | 99.99% w/w |
| 12. | Residual solvents (By GC) | Methanol : NMT 1000ppm IPA : NMT 1000ppm |  | 49.84 ppm 95.21 ppm |
| 13. | Particle size (By Malvern Dry Method) | For information only |  | 100% Particles less than 15 µm |
| REMARK :THE PRODUCT CONFORMS TO ABOVE SPECIFICATIONS. Compiled by: Checked by: Authorized signatory: |  |  |  |  |