Certificate of Analysis

| Product Name： | Metronidazole |  | Packing： |  | 25 kg/drum |
| --- | --- | --- | --- | --- | --- |
| Batch No.： | 0182605073 |  | Manufacturing Date： |  | 2026-05-07 |
| Batch Quantity： | 1124.05 kg |  | Test Date： |  | 2026-05-09 |
| Storage | Stored in closed container and protected from light |  | Expire Date： |  | 2030.04 |
| Test Specification： BP-USP |  |  |  |  |  |
| Test Items |  | Acceptance criteria |  | Tested Results |  |
| Appearance |  | white or yellowish, crystalline powder |  | Conforms |  |
| Identification |  |  |  |  |  |
| Melting point(℃) |  | 159～163 |  | 160.9~161.2 |  |
| IR |  | The sample spectrum complies with that of the reference standard spectrum. |  | Conforms |  |
| UV |  | The solution shows an absorption maximum at |  | Conforms |  |
| Reaction of primary aromatic amines |  | 277nm, and a minimum at 240. The specific absorption at the maximum is 365 to 395. The sample solution gives the reaction of prima1y aromatic amines. |  | Conforms |  |
| Related Substances (HPLC) Tindazole related compound A |  | ≤0.01 |  | 0.01 |  |
| Any single unspecified impurity,% |  | ≤0.01 |  | 0.02 |  |
| Total Impurities,% |  | ≤0.2 |  | 0.03 |  |
| Loss on drying(%) |  | ≤0.5 |  | 0.2 |  |
| Heavy metals(ppm) |  | ≤20 |  | Conforms |  |
| Reside on ignition |  | ≤0.1 |  | 0.06 |  |
| Microbiology test(cfu/g) |  | Total aerobic microbial count (TAMC) should be no more than 103 in 1 g |  | <1x10 |  |
|  |  | Total Yeasts/Moulds count (TYMC) should be no more than 102 in 1 g |  | <1x10 |  |
|  |  | Absence of Escherichia coli in 1 g |  | N.D |  |
| Residual Solvents Methyl alcohol (ppm) Methyl formate (ppm) Ethylene oxide (ppm) |  | <3000 <681 <1 |  | 27.77 N.D N.D |  |
| Assay (calculated on the dried substance),% |  | 99.0～101.0 |  | 99.6 |  |
| Bacterial endotoxin (EU/mg) |  | <0.35 |  | Conforms |  |
| Note: N.D indicates “Not detected“ |  |  |  |  |  |

Conclusion: This batch of metronidazole complies with the requirement of the specifications above.

| Reporter/Date： | Reviewer/Date： | QA Head/Date： |
| --- | --- | --- |