| Product Name.: MAGNESIUM STEARATE | Batch No: MG-01A022022 |
| --- | --- |
| Mfg. Date : FEB-2022 | Expiry Date: FEB-2027 |
| Date: 24/03/2022 | Grade: Pharma (USP/BP/EP) |

CERTIFICATE OF ANALYSIS

| TEST | SPECIFICATION | RESULT |
| --- | --- | --- |
| Identification Method |  |  |
| Identification Test A-(BP,EP) | Freezing Point-53°C Min | 58°C |
| Identification Test C(BP) | Retention time of the principal peaks in the chromatogram obtained with the test solution are approximately the same as those of the principal peaks in the chromatogram obtained with the reference solution | Complies |
| Identification Test A-(USP) | This solution responds to the test for Magnesium | Complies |
| Identification Test B-(USP) | Retention time of Stearic and palmitic acid Match standard. | Complies |
| Identification Test D (BP) | White crystalline precipitate is formed | Complies |
| Acid value Of Fatty Acid | 195-210 | 204 |
| ASSAY (BP/EP/USP) As Mg | 4-5 % on dried basis | 4.38 |
| Acidity/ Alkalinity(USP/BP) | NMT 0.05 ml of 0.1 N hydrochloric acid or 0.1 N sodium hydroxide is required to change the color of the indicator. | Complies |
| Loss on Drying (USP/BP) | Max 6.0% | 2.8 |
| Fatty Acid (USP/BP) |  |  |
| Stearic Acid | NLT 40% | Complies |
| Sum of Palmitic Acid + Stearic acid | NLT 90% | Complies |
| Nickel (BP) | PPM Max 5.0 | 1.6 |
| Cadmium (BP) | PPM Max 3.0 | 0.84 |
| Heavy Metal As Lead-(%) | 0.002 | <0.002 |
| Chloride (USP/BP/EP) | Max 0.1% | <0.1 |
| Microbial Contamination (USP) | TAMC-NMT 10³,TYMC-NMT 5x10², Salmonella & Escherichia Coli-Absent | Complies |
| Microbial Contamination (BP) | TAMC-NMT 10³,TYMC-NMT 10², Salmonella & Escherichia Coli-Absent | Complies |
| Specific Surface area (BP) | 3.0-14.0 m²/gm | 9.96 |
| Additional test |  |  |
| Assay as Mgo (%) | 6.8-8.3 | 7.3 |
| Free Fatty Acid (%) | 1.0 Max | 0.72 |
| Bulk density (g/L) | 250-320 | 263 |
| Melting Point ºC | 130-150 | 132 |
| Particle Size (thru 325 mesh) | 99% particle passes from 325 mesh | 99.8 |
| Pseudomonas aeruginosa | Negative | Complies |
| Staphylococcus aureus | Negative | Complies |
| Lead (ppm) | 2.0 Max | <2.0 |
| Arsenic As AS2o3 (%) | 0.0002 Max | <0.0002 |
| Sulphate (%) | 0.3 Max | <0.3 |
| Sum of methyl Stearate And palmitate (%) | 90 Min | Passed |
| Methyl Stearate (%) | 40 Min | Passed |
| Organic Volatile impurities (ppm) | Passed | Passed |
| Benzene (ppm) | 2 Max | Not Detected |
| Chloroform (ppm) | 50 Max | Not Detected |
| 1,4-Dioxane (ppm) | 100 Max | Not Detected |
| Methylene Chloride (ppm) | 500 Max | Not Detected |
| Trichloroethylene (ppm) | 80 Max | Not Detected |
| Clarity of Fatty Acid | Clear | Complies |
| Freezing Point (C) | 54 Min | 58 |
| Remarks: It occurs as a white, light bulky powder. It is smooth to the touch and sticky to the skin. It has no odor or a faint, characteristic odor. It is practically insoluble in water and Ethanol (95). |  |  |