| CERTIFICATE OF ANALYSIS |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Name |  | : | Betamethasone Dipropionate USP Micronised |  | Batch Qty. |  | : | 1.500 Kgs |
| Batch No |  | : | BDPFP24005 |  | A.R. No. |  | : | AFP/912/2024 |
| Mfg. Date |  | : | 07/2024 |  | Exp. Date |  | : | 06/2029 |
| Sr.No. | Test |  |  | Observation |  | Limit |  |  |
| 1 | Description |  |  | Complies |  | White to cream-white powder odourless powder. |  |  |
| 2 | Solubility |  |  | Complies |  | Freely soluble in acetone and in chloroform, sparingly soluble in alcohol; insoluble in water. |  |  |
| 3 | Identification- |  |  |  |  |  |  |  |
|  | - IR - TLC |  |  | Complies Complies |  | IR spectrum of the sample dispersed shall be concordant with that of the working standard. The principal spot in the chromatogram obtained with the test solution corresponds to the principal spot in the chromatogram obtained with standard solution. |  |  |
| 4 | Specific optical rotation |  |  | +65° |  | Between+63° and +70° |  |  |
| 5 | Loss on drying |  |  | 0.32% |  | NMT 1.0% w/w |  |  |
| 6 | Residue On Drying |  |  | 0.04% |  | NMT 0.2% w/w |  |  |
| 7 | Organic Impurity (By HPLC) |  |  |  |  |  |  |  |
|  | Individual impurity |  |  | 0.31% |  | Not more than 1.0% |  |  |
|  | Total impurity |  |  | 0.56% |  | Not more than 2.0% |  |  |
| 8 | Assay (By HPLC) |  |  | 99.2% |  | Between 97.0% and 103.0% w/w Calculated on dried basis. |  |  |
| 9 | Residual Solvents: |  |  |  |  |  |  |  |
|  | Methylene chloride |  |  | ND |  | NMT 600 ppm |  |  |
|  | Acetone |  |  | 148 ppm |  | NMT 5000 ppm |  |  |
|  | Ethyl Acetate |  |  | ND |  | NMT 5000 ppm |  |  |
|  | Methanol |  |  | 54 ppm |  | NMT 3000 ppm |  |  |
|  | Pyridine |  |  | ND |  | NMT 200 ppm |  |  |
|  | DMF |  |  | ND |  | NMT 880 ppm |  |  |
| 10 | Additional Test: |  |  |  |  |  |  |  |
|  | Particle size |  |  | Complies |  | 90% less than 10 micron |  |  |
|  | Particle size |  |  | Complies |  | 100% less than 20 micron |  |  |
| Remarks: The material complies as per prescribed USP specification. |  |  |  |  |  |  |  |  |
| For Anuh Pharma Limited, Authorised Signatory Date: 23/07/2024. |  |  |  |  |  |  |  |  |